Algorithm change IMEI on x55 x60 models.

The x55 x60 models have flash chip with 128 bit OTP zone, 64 bit – Manafacturer (only read), 64 –bit user. In 64 bit user register stored OTP IMEI. On Siemens Factory  programming 64 bit OTP user registers and lock it. After this you cannot change 64 bit User register (OTP IMEI) .

Siemens firmware have function checking OTP IMEI with EEPROM IMEI. If IMEIs not same phone is not start only beep and exit with error E002. This Error he send to port on baudrate 57600. For change IMEI we need dissable OTP IMEI Checking.

This part Firmware M55 v10 phone disassemled on IDA 4.5 for undestanding how get and checking IMEI:

Procedure Extract_IMEI:

B7000:3A1A ExtractIMEI:                            ; CODE XREF: sub_9E9A9E+1CP

B7000:3A1A                                         ; sub_A101B0+FEP

B7000:3A1A                                         ; sub_A1030E+3AP

B7000:3A1A                                         ; sub_A10FC8+5CP

B7000:3A1A                                         ; sub_B75A00+88P

B7000:3A1A                 mov     [-r0], r8

B7000:3A1C                 sub     r0, #0Ah

B7000:3A20                 mov     r12, r0

B7000:3A22                 and     r12, #3FFFh

B7000:3A26                 mov     r13, 0FE02h ; 0C000FE02h

B7000:3A2A                 calls   0BFh, Check_IMEI  : In this function we compare IMEI`s

B7000:3A2E                 mov     r8, r4

B7000:3A30                 jmpr    cc_Z, loc_B73A80

B7000:3A32                 cmp     r8, #4

B7000:3A34                 jmpr    cc_Z, loc_B73A80

B7000:3A36                 cmp     r8, #2

B7000:3A38                 jmpr    cc_Z, loc_B73A3E

B7000:3A3A                 cmp     r8, #3

B7000:3A3C                 jmpr    cc_NZ, loc_B73A48

B7000:3A3E loc_B73A3E:                             ; CODE XREF: ExtractIMEI+1Ej

B7000:3A3E                 calls   0A1h, sub_A10C1E

B7000:3A42                 cmp     r4, #0

B7000:3A44                 jmpr    cc_NZ, loc_B73A80

B7000:3A46                 jmpr    cc_UC, loc_B73A72

B7000:3A48 ; ---------------------------------------------------------------------------

In this part we sending ERROR :E002

B7000:3A48 loc_B73A48:                             ; CODE XREF: ExtractIMEI+22j

B7000:3A48                 calls   0A1h, sub_A10C1E

B7000:3A4C                 cmp     r4, #0

B7000:3A4E                 jmpr    cc_NZ, loc_B73A5E

B7000:3A50                 and     r8, #0Fh

B7000:3A54                 or      r8, #0E000h

B7000:3A58                 mov     0FE8Eh, r8 ; 0C000FE8Eh

B7000:3A5C                 trap    #30h ; '0'   - Exit Exceptin   Trap 30h

B7000:3A5E loc_B73A5E:                             ; CODE XREF: ExtractIMEI+34j

B7000:3A5E                 mov     r12, #(IMEIExtracted - unk_13C000)

B7000:3A62                 mov     r13, #4Fh ; 'O'

B7000:3A66                 mov     r14, #0

B7000:3A68                 mov     r15, #8

B7000:3A6A                 calls   0BFh, Memset    ; R15_size R14_fill_with

B7000:3A6E                 jmpa    cc_UC, loc_B73B08

B7000:3A72 ; ---------------------------------------------------------------------------

B7000:3A72 loc_B73A72:                             ; CODE XREF: ExtractIMEI+2Cj

B7000:3A72                 and     r8, #0Fh

B7000:3A76                 or      r8, #0E000h

B7000:3A7A                 mov     0FE8Eh, r8 ; 0C000FE8Eh

B7000:3A7E                 trap    #30h ; '0'

B7000:3A80 loc_B73A80:                             ; CODE XREF: ExtractIMEI+16j

B7000:3A80                                         ; ExtractIMEI+1Aj

B7000:3A80                                         ; ExtractIMEI+2Aj

B7000:3A80                 movb    rl1, [r0+7]

B7000:3A84                 movbz   r1, rl1

B7000:3A86                 shr     r1, #4

B7000:3A88                 movb    0B43Eh, rl1 ; 0C000F43Eh

B7000:3A8C                 mov     DPP0, #4Fh ; 'O'

B7000:3A90                 movb    rl2, #0FAh ; '·'

B7000:3A94                 movb    3FE8h, rl2 ; 0C100FFE8h

B7000:3A98                 mov     r12, #0

B7000:3A9A                 mov     r13, #1

B7000:3A9C loc_B73A9C:                             ; CODE XREF: ExtractIMEI+CEj

B7000:3A9C                 mov     r8, r13

B7000:3A9E                 and     r8, #1

B7000:3AA0                 jmpr    cc_NZ, loc_B73AB8

B7000:3AA2                 mov     r8, r12

B7000:3AA4                 add     r8, r0

B7000:3AA6                 movb    rl1, [r8]

B7000:3AA8                 movbz   r1, rl1

B7000:3AAA                 shr     r1, #4

B7000:3AAC                 extp    #4Fh, #1 ; 'O'

B7000:3AB0                 movb    [r12+(unk_13FFE9 - unk_13C000)], rl1

B7000:3AB4                 add     r12, #1

B7000:3AB6                 jmpr    cc_UC, loc_B73AE2

B7000:3AB8 ; ---------------------------------------------------------------------------

B7000:3AB8 loc_B73AB8:                             ; CODE XREF: ExtractIMEI+86j

B7000:3AB8                 mov     r14, #(IMEIExtracted - unk_13C000)

B7000:3ABC                 mov     r15, #4Fh ; 'O'

B7000:3AC0                 add     r14, r12

B7000:3AC2                 extp    r15, #1

B7000:3AC4                 movb    rl1, [r14]

B7000:3AC6                 andb    rl1, #0Fh

B7000:3ACA                 extp    r15, #1

B7000:3ACC                 movb    [r14], rl1

B7000:3ACE                 mov     r8, r12

B7000:3AD0                 add     r8, r0

B7000:3AD2                 movb    rl1, [r8]

B7000:3AD4                 movbz   r1, rl1

B7000:3AD6                 shl     r1, #4

B7000:3AD8                 extp    r15, #1

B7000:3ADA                 movb    rl2, [r14]

B7000:3ADC                 orb     rl2, rl1

B7000:3ADE                 extp    r15, #1

B7000:3AE0                 movb    [r14], rl2

B7000:3AE2 loc_B73AE2:                             ; CODE XREF: ExtractIMEI+9Cj

B7000:3AE2                 add     r13, #1

B7000:3AE4                 cmp     r13, #0Fh

B7000:3AE8                 jmpr    cc_C, loc_B73A9C

B7000:3AEA                 mov     r8, r12

B7000:3AEC                 add     r8, r0

B7000:3AEE                 movb    rl1, [r8]

B7000:3AF0                 movbz   r1, rl1

B7000:3AF2                 shl     r1, #4

B7000:3AF4                 mov     r8, #(IMEIExtracted - unk_13C000)

B7000:3AF8                 mov     r13, #4Fh ; 'O'

B7000:3AFC                 add     r8, r12

B7000:3AFE                 extp    r13, #1

B7000:3B00                 movb    rl2, [r8]

B7000:3B02                 orb     rl2, rl1

B7000:3B04                 extp    r13, #1

B7000:3B06                 movb    [r8], rl2

B7000:3B08 loc_B73B08:                             ; CODE XREF: ExtractIMEI+54j

B7000:3B08                 add     r0, #0Ah

B7000:3B0C                 mov     r8, [r0+]

B7000:3B0E                 rets

B7000:3B0E ; End of function ExtractIMEI

This function in Procedure Extract_IMEI. We need for dissabling OTP IMEI checking.

Function Compare IMEI`s OTP and EEPROM.

BF000:84A4 Check_IMEI:                             ; CODE XREF: ExtractIMEI+10P

BF000:84A4                 mov     [-r0], r9

BF000:84A6                 mov     [-r0], r8

BF000:84A8                 sub     r0, #0Ah

BF000:84AC                 mov     r8, r12

BF000:84AE                 mov     r9, r13

BF000:84B0                 calls   0BFh, GetOTPIMEI

BF000:84B4                 cmp     r4, #0

BF000:84B6                 jmpr    cc_Z, loc_BF8516

BF000:84B8                 mov     r12, r0

BF000:84BA                 and     r12, #3FFFh

BF000:84BE                 mov     r13, DPP1

BF000:84C2                 calls   0BFh, GET_IMEI_BLK2

BF000:84C6                 cmp     r4, #0

BF000:84C8                 jmpr    cc_Z, Compare_IMEI`s

BF000:84CA                 cmp     r4, #4

BF000:84CC                 jmpr    cc_Z, loc_BF84F2

BF000:84CE                 jmpr    cc_UC, loc_BF8510

BF000:84D0 ; ---------------------------------------------------------------------------

Zone for patching IMEI:

BF000:84D0 Compare_IMEI`s:                         ; CODE XREF: Check_IMEI+24j

BF000:84D0                 mov     r12, #8

BF000:84D2                 mov     [-r0], r12

BF000:84D4                 mov     r12, #2

BF000:84D6                 add     r12, r0

BF000:84D8                 and     r12, #3FFFh

BF000:84DC                 mov     r13, DPP1

BF000:84E0                 mov     r14, r8

BF000:84E2                 mov     r15, r9

BF000:84E4                 calls   0BFh, MemCompare ; R0 = Size [R13:R12]<=>[R15:R14] return r4

BF000:84E8                 add     r0, #2

BF000:84EA                 cmp     r4, #0

BF000:84EC                 jmpr    cc_NZ, IMEI_Is_Not_SAME

BF000:84EE                 mov     r12, #0

BF000:84F0                 jmpr    cc_UC, loc_BF8512

END of  zone  Patching IMEI.

BF000:84F2 ; ---------------------------------------------------------------------------

BF000:84F2 loc_BF84F2:                             ; CODE XREF: Check_IMEI+28j

BF000:84F2                 mov     r12, #8

BF000:84F4                 mov     [-r0], r12

BF000:84F6                 mov     r12, #2

BF000:84F8                 add     r12, r0

BF000:84FA                 and     r12, #3FFFh

BF000:84FE                 mov     r13, 0FE02h ; 0C000FE02h

BF000:8502                 mov     r14, r8

BF000:8504                 mov     r15, r9

BF000:8506                 calls   0BFh, MEMcopy   ; from [R15:R14]  to [R13:R12]  r0 - counter

BF000:850A                 add     r0, #2

BF000:850C                 mov     r12, #3

BF000:850E                 jmpr    cc_UC, loc_BF8512

BF000:8510 ; ---------------------------------------------------------------------------

BF000:8510 IMEI_Is_Not_SAME:                             ; CODE XREF: Check_IMEI+2Aj

BF000:8510                                         ; Check_IMEI+48j

BF000:8510                 mov     r12, #2

BF000:8512 loc_BF8512:                             ; CODE XREF: Check_IMEI+4Cj

BF000:8512                                         ; Check_IMEI+6Aj

BF000:8512                 mov     r4, r12

BF000:8514                 jmpr    cc_UC, loc_BF851E

BF000:8516 ; ---------------------------------------------------------------------------

BF000:8516 loc_BF8516:                             ; CODE XREF: Check_IMEI+12j

BF000:8516                 mov     r12, r8

BF000:8518                 mov     r13, r9

BF000:851A                 calls   0BFh, GET_IMEI_BLK2

BF000:851E loc_BF851E:                             ; CODE XREF: Check_IMEI+70j

BF000:851E                 add     r0, #0Ah

BF000:8522                 mov     r8, [r0+]

BF000:8524                 mov     r9, [r0+]

BF000:8526                 rets

BF000:8526 ; End of function Check_IMEI

For patching zone compare IMEI`s you need make this changes:

Zone for patching IMEI:

| Normal : | Patched: |
| --- | --- |
| BF000:84D0 Compare_IMEI`s: |  |
| BF000:84D0 mov r12, #8 |  |
| BF000:84D2 mov [-r0], r12 |  |
| BF000:84D4 mov r12, #2 | mov r14, #2 |
| BF000:84D6 add r12, r0 | add r14, r0 |
| BF000:84D8 and r12, #3FFFh | and r14, #3FFFh |
| BF000:84DC mov r13, DPP1 | mov r15, DPP1 |
| BF000:84E0 mov r14, r8 | mov r12, r8 |
| BF000:84E2 mov r15, r9 | mov r13, r9 |
| BF000:84E4 calls 0BFh, MemCompare ; R0 | calls 0BFh, MEMcopy |
| BF000:84E8 add r0, #2 | nop |
| BF000:84EA cmp r4, #0 | nop |
| BF000:84EC jmpr cc_NZ, IMEI_Is_Not_SAME |  |
| BF000:84EE mov r12, #0 |  |
| BF000:84F0 jmpr cc_UC, loc_BF8512 |  |

END of  zone  Patching IMEI.

This patch make changes from Compare IMEI`s in RAM  to Copy EEPROM IMEI in RAM to OTP IMEI in RAM.

Patching in HEX:

Offset                                                                                            Call

Old          -  9F84D4:          E02C00C066FCFF3FF2FD02FEF0E8F0F9 DABFDEA8 080248403D11

Patched   -  9F84D4:          E02E00E066FEFF3FF2FF02FEF0C8F0D9 DABF8CA7 0802CC00CC00

Offset- Offset from start fIrmware

Call – Call  BFA8DE (MEMCompare) and Call BFA78C (MEMCopy)

In Call we make changes Calling fuction from MEMCompare to MEMCopy.

For find Procedure we are searching in Disaassembler IDA. This part of procedure Extract_IMEI:

In asm:    B7000:3A76                 or      r8, #0E000h

In HEX:                                            76F800E0

After find we are disassembling and entring to function Compare_IMEI and patching this function. For find start address zone for patching and address function MEMCopy. And we appying patch.

After add patch use Frea and change IMEI to any dights. And phone perfect.

How to appy Patch.

- You need V_klay or Hex edit.

Instruction for appy patch on V_Klay:

Select Model, Port, BaudRate. Press Open Button. And select vkp File with patch. And press appy patch buuton. After patching Exit program. Run Frea and change IMEI.

Siemens C55 have CRC checking need disable,  patch C55 have CRC patch and  IMEI Change patch.

Siemens A60 have wrong addressing in V_klay you need read full flash start 0x800000 or read open in HEX Edit and appy changes to offset.

All other phones patching only add patch and change IMEI on Frea.

If you want you can make your software in folder “ Source” Source code Frea, V_klay.

In Folder “IDA” - IDA 4.3

In Folder “Firmware” – Disassemled firmware M55 v10 with opened Procedures for IMEI.

In folder “Software” – Frea all version.

V_klay you can download from: http://vi-soft.com.ua

------

Best Regards,

Andrey Musaev